INGLETON SWIMMING POOL

RISK ASSESSMENT PLAN

| HAZARD AND RELATED ACTIVITIES e.g. trip, falling objects, fire, explosion, noise, chemicals, violence | PERSONS AT RISK e.g. employees, pupils, customers, contractors, members of public, other | EXISTING CONTROLS e.g. Guards, Workplace Instructions, Training, Authorised Users, Competent Persons, Personal Protective Equipment (PPE), GIVE FULL DETAILS | POSSIBLE OUTCOME |  | RESIDUAL RISK RATING AFTER EXISTING CONTROLS (if 16 or more and no further controls practicable add to the H&S Risk Register) | ADDITIONAL CONTROLS REQUIRED? | RESIDUAL RISK RATING AFTER ADDITIONAL CONTROLS (if 16 or more add to the H&S Risk Register; if 21+ STOP ACTIVITY) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Potential Harm | Likelihood |  |  |  |
| Risk of transmitting or contracting Coronavirus | Customers and staff | The Pool did not open during the 2020 season due to the Coronavirus epidemic. | Minor | Possible | 9 | 2023 Season – Management Committee decision to remove all COVID-19 guidelines as they are no longer required by law. No guidance has been incorporated into the NOP. 2024 Season – no requirement by law to implement any COVID-19 guidelines, but an awareness of the virus causing illness in staff and customers. As an outdoor venue transmission low risk due. Continue with regular disinfection of changing rooms and other communal areas as per normal operating procedures and hygiene routines. Same applies for 2025 and 2026 season. | 9 |
| Drowning - General Public Session, specific activity/event sessions and Private Hire: Customers may become unconscious or cannot swim properly Pool Staff as the result of a rescue | Customers and Staff | - Only NPLQ Lifeguards (or equivalent) are used by the Swimming Pool. - Lifeguards are in attendance at every General Public Session - Lifeguards are in attendance at every Private Hire session and specific activity/event session. - Pool depth and safety signs are displayed - Life Saving Equipment available at the pool side - A portable defibrillator is available at the pool side when the pool is open. Lifeguards and pool supervisor staff are trained in its use. | Major | Possible | 19 | Purchase of a PXB board has been considered (which can be potentially be used by 1 lifeguard only). However, discussion with an RLSS advisor (15.05.2018) confirmed as our pool is freeboard and not deckside, the use of the PXB board in our setting would require 2 lifeguards plus another trained member of staff. Review of our Accident Book does not identify a significant risk of an incident occurring where a PXB board would be needed. 2024 Season – new buoys and throw ropes purchased. One buoy available to be carried per lifeguard. 2025 and 2026 season – poolside equipment checked each year and replaced as necessary before the start of the season. | 19 |

| HAZARD AND RELATED ACTIVITIES |  | PERSONS AT RISK |  | EXISTING CONTROLS | POSSIBLE OUTCOME |  |  |  | RESIDUAL RISK RATING AFTER EXISTING CONTROLS |  | ADDITIONAL CONTROLS REQUIRED? |  | RESIDUAL RISK RATING AFTER ADDITIONAL CONTROLS |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Potential Harm |  | Likelihood |  |  |  |  |  |  |  |
| Drowning - Adult Member Sessions: Customers may become unconscious or cannot swim properly Pool Staff as the result of a rescue | Customers and staff |  | - Lifeguards are in attendance at every Adult Member Session - Pool depth and safety signs are displayed - Life Saving Equipment available at the pool side - A portable defibrillator is available at the pool side when the pool is open. Lifeguards and pool staff are trained in its use. |  |  | Major |  | Possible |  | 19 |  | Purchase of a PXB - s ee notes in preceding section. The requirements of HSG 179 in respect of unsupervised swimming have been considered previously and use of additional drowning detection technology or specific CCTV monitoring was not considered to be justified or appropriate to our setting. This consideration is now superseded as Adult Member sessions are now all supervised by a qualified lifeguard. |  | 19 |

| HAZARD AND RELATED ACTIVITIES | PERSONS AT RISK | EXISTING CONTROLS | POSSIBLE OUTCOME |  | RESIDUAL RISK RATING AFTER EXISTING CONTROLS | ADDITIONAL CONTROLS REQUIRED? | RESIDUAL RISK RATING AFTER ADDITIONAL CONTROLS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Potential Harm | Likelihood |  |  |  |
| External Distraction: Staff can be distracted by what is going on around them and they may miss an incident in the pool | Customers and Staff | - Only NPLQ Lifeguards (or equivalent) are used by the Swimming Pool. - Lifeguards patrol poolside to help reduce the likelihood of being distracted. - Lifeguards must not allow themselves to be distracted at all by colleagues or members of the public - Professional conversations as listed in the NOP (less than 20 seconds) | Moderate | Possible | 14 | No mobile phones on poolside. No headphones to be worn whilst on duty – either on poolside or in the office whilst swimmers are on the premises. | 14 |
| Solar glare/reflection: Because we are an outdoor pool glare and reflection prevents the Lifeguard from watching the pool effectively | Customers and Staff | - Only NPLQ Lifeguards (or equivalent) are used by the Swimming Pool. - Lifeguards patrol poolside to help reduce the likelihood of glare/reflection. They are experienced and know the best observation positions for the weather conditions. | Moderate | Possible | 14 | Lifeguards must be aware of the different zones of visibility with the changing position of the sun throughout a day and changing light levels. | 14 |

| HAZARD AND RELATED ACTIVITIES | PERSONS AT RISK | EXISTING CONTROLS | POSSIBLE OUTCOME |  | RESIDUAL RISK RATING AFTER EXISTING CONTROLS | ADDITIONAL CONTROLS REQUIRED? | RESIDUAL RISK RATING AFTER ADDITIONAL CONTROLS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Potential Harm | Likelihood |  |  |  |
| Unauthorised Access: Members of the general public gaining illegal access to the swimming pool area outside of normal operating hours | General Public | - The pool is surrounded by a high fence which is regularly checked and maintained. The fence is fit for purpose. - An Emergency Gate within the pool outer fence is kept locked. A key is available via an Emergency Key Box. - The only other entrance to the pool is by the Pool Kiosk and is locked when the pool is not in use. - All buildings are locked when the pool is closed. - CCTV is in operation to cover out of hours. | Moderate | Possible | 14 |  | 14 |

| HAZARD AND RELATED ACTIVITIES | PERSONS AT RISK | EXISTING CONTROLS | POSSIBLE OUTCOME |  | RESIDUAL RISK RATING AFTER EXISTING CONTROLS | ADDITIONAL CONTROLS REQUIRED? | RESIDUAL RISK RATING AFTER ADDITIONAL CONTROLS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Potential Harm | Likelihood |  |  |  |
| Pool Access Ladders: Customers slipping on entry and exit, getting trapped in the rungs or between the ladder and the pool wall | Customers and Staff | - Daily inspections of the ladders and steps to ensure that the handrails and steps are secure and that there are no sharp edges or damaged steps - Non-slip steps are used on the ladders - Ladders have wide steps - The space between the pool wall and the steps is kept to a minimum | Moderate | Unlikely | 13 |  | 13 |
| Maximum Bather load: | Customers and Staff | - The maximum bather loads are stipulated within the NOP, under no circumstances can they be exceeded. - All staff must follow the overcrowding EAP in the NOP. - An online booking system has been introduced since 2021 to reduce overcrowding. | Moderate | Unlikely | 13 |  | 13 |
| HAZARD AND RELATED ACTIVITIES | PERSONS AT RISK | EXISTING CONTROLS | POSSIBLE OUTCOME |  | RESIDUAL RISK RATING AFTER EXISTING CONTROLS | ADDITIONAL CONTROLS REQUIRED? | RESIDUAL RISK RATING AFTER ADDITIONAL CONTROLS |
|  |  |  | Potential Harm | Likelihood |  |  |  |
| Staff under the influence of drink/drugs: This includes staff that are hung-over and/or smelling of alcohol | Customers and Staff | - This is a serious safety issue and no member of staff is allowed on duty if they are suspected taking medication that may hinder their performance at work. - Staff must be sent home and they must be reported as unfit for work, due to drink/drugs. They cannot drive home. - A member of staff will inform the Pool Management Committee without delay. - All staff are aware of the pool’s no alcohol policy as detailed in the NOP. | Moderate | Unlikely | 13 |  | 13 |

| HAZARD AND RELATED ACTIVITIES | PERSONS AT RISK | EXISTING CONTROLS | POSSIBLE OUTCOME |  | RESIDUAL RISK RATING AFTER EXISTING CONTROLS | ADDITIONAL CONTROLS REQUIRED? | RESIDUAL RISK RATING AFTER ADDITIONAL CONTROLS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Potential Harm | Likelihood |  |  |  |
| Sump Grid/s: Customers may get trapped playing on or around the sump grid | Customers | - Pool design incorporates all safety measures on inlets and outlets - Inlets and outlets of the pool circulation system have suitable covers to prevent limbs and fingers getting trapped - Multiple inlets and outlets and inlets reduce the risk of undue suction which could result in a body being held against a grill - There are no exposed sharp edges on the inlets and outlets - Inlets/outlets are inspected daily for any damage | Major | Remote | 12 |  | 12 |

| HAZARD AND RELATED ACTIVITIES | PERSONS AT RISK | EXISTING CONTROLS | POSSIBLE OUTCOME |  | RESIDUAL RISK RATING AFTER EXISTING CONTROLS | ADDITIONAL CONTROLS REQUIRED? | RESIDUAL RISK RATING AFTER ADDITIONAL CONTROLS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Potential Harm | Likelihood |  |  |  |
| Customers under the influence of drink/drugs: Customers may attempt to swim whilst drunk/hungover or whilst on prescription or illegal drugs. | Customers and Staff | - Pool Supervisor or Pool Assistants advises the Lifeguard if they suspect customers are intoxicated or under the influence (who restricts access to the pool). The Pool Supervisor or Pool Assistants can refuse entry to the customer if they feel there is a risk to Staff and other customers. - Staff are authorised to call the police (999) if customers under the influence of drugs/alcohol refuse to leave the premises when asked to do so | Major | Remote | 12 |  | 12 |

| HAZARD AND RELATED ACTIVITIES | PERSONS AT RISK | EXISTING CONTROLS | POSSIBLE OUTCOME |  | RESIDUAL RISK RATING AFTER EXISTING CONTROLS | ADDITIONAL CONTROLS REQUIRED? | RESIDUAL RISK RATING AFTER ADDITIONAL CONTROLS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Potential Harm | Likelihood |  |  |  |
| Pool water clarity: A chemical imbalance or air can reduce the visibility in the water making it difficult to see effectively. Air can also block or restrict the water flow making the pool unsafe and unhygienic | Customers and Staff | - The pool’s levels are controlled by automatic valves which are subjected to annual inspections - Only trained Pool Staff or volunteers who have attended the National Pool Plant Operators Certificate course can adjust the valves, the most likely cause of an air blockage. - Staff must follow the EAP for Lack of Water Clarity without delay - No swimming is allowed if the whole of the pool floor cannot be seen, - Lifeguards, Pool Supervisor and Pool Assistants are authorised to close the pool, if the water clarity is too poor or deemed unsafe. | Major | Remote | 12 |  | 12 |

| HAZARD AND RELATED ACTIVITIES | PERSONS AT RISK | EXISTING CONTROLS | POSSIBLE OUTCOME |  | RESIDUAL RISK RATING AFTER EXISTING CONTROLS | ADDITIONAL CONTROLS REQUIRED? | RESIDUAL RISK RATING AFTER ADDITIONAL CONTROLS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Potential Harm | Likelihood |  |  |  |
| Pool water clarity: Continued | Customers and Staff | - Pool Staff and Lifeguards are trained in Water Quality testing procedures for Chlorine and pH levels. - Swimming pool water is tested at the start and end of every day and at set times during the day for chemical levels, clarity and temperature and results are recorded on a pre-printed form (Water Quality Log) and kept in the Pool Office – records are kept for 5 years. - Additional Weekly Water Balance tests are performed and recorded on a pre-printed form (Water Quality Log) and kept in the Pool Office – records are kept for 5 years. | Major | Remote | 12 |  | 12 |

| HAZARD AND RELATED ACTIVITIES | PERSONS AT RISK | EXISTING CONTROLS | POSSIBLE OUTCOME |  | RESIDUAL RISK RATING AFTER EXISTING CONTROLS | ADDITIONAL CONTROLS REQUIRED? | RESIDUAL RISK RATING AFTER ADDITIONAL CONTROLS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Potential Harm | Likelihood |  |  |  |
| Pool water clarity: Continued | Customers and Staff | - Swimming pool water is regularly sent for quality analysis to an independent laboratory. - Reactive Water Quality maintenance is continually undertaken as the result of the above monitoring. - Specialist contractors are regularly brought in for regular service of pool water treatment plant and equipment. | Major | Remote | 12 |  | 12 |
| Fluctuating water levels: The pool’s level can fall or rise creating either an impact hazard or slipping hazard (if too high) | Customers and Staff | - The pool’s level is controlled by automatic valves which are subjected to annual inspections. - Backwashes are done when the pool is closed. | Water level too low: Major Water level too high: Insignificant | Water level too low: Remote Water level too high: Unlikely | Water level too low: 12 Water level too high: 3 |  | 12 3 |

| HAZARD AND RELATED ACTIVITIES | PERSONS AT RISK | EXISTING CONTROLS | POSSIBLE OUTCOME |  | RESIDUAL RISK RATING AFTER EXISTING CONTROLS | ADDITIONAL CONTROLS REQUIRED? |  | RESIDUAL RISK RATING AFTER ADDITIONAL CONTROLS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Potential Harm | Likelihood |  |  |  |  |
| Customers with specific needs: | Customers and Staff | - Specific needs are determined through direct consultation with the individual or parent by Pool Staff to determine if it can be provided. If not then entry will not be allowed. - Wheelchair access ramp to the pool entrance - There is a dedicated changing facility with all necessary requirements including a shower. - Disabled access steps at the shallow end of the pool. | Minor | Probable | 10 |  |  | 10 |
| Effects of the sun: Due to the outdoor nature of the pool and that it is a summer facility, Lifeguards can become dehydrated and suffer from the effects of the sun | Staff | - Lifeguards are issued with a hat. - Lifeguards are encouraged to keep well hydrated. - Lifeguards are encouraged to use sun cream. - Umbrellas available to provide shade | Minor | Possible | 9 |   | Ensure sun cream is available in the Pay Kiosk | 9 |

| HAZARD AND RELATED ACTIVITIES | PERSONS AT RISK | EXISTING CONTROLS | POSSIBLE OUTCOME |  | RESIDUAL RISK RATING AFTER EXISTING CONTROLS | ADDITIONAL CONTROLS REQUIRED? | RESIDUAL RISK RATING AFTER ADDITIONAL CONTROLS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Potential Harm | Likelihood |  |  |  |
| Wet pool surrounds: Due to the nature of the pool’s activities, customers can wet the poolside by getting out, this is unavoidable | Customers and Staff | - No running on poolside - None slip floor surface fitted around the pool - The pool side surface is inspected every night - The pool surround is kept clear of potential hazards by staff - Any spillages are cleared immediately (access to a hosepipe) | Minor | Possible | 9 |  | 9 |
| Changing Facilities Entrance: Due to the open nature of the changing rooms and the proximity to the showers children may jump straight into the pool | Customers |  Lifeguards watch the area as part of their patrols | Minor | Possible | 9 |  | 9 |

| HAZARD AND RELATED ACTIVITIES | PERSONS AT RISK | EXISTING CONTROLS | POSSIBLE OUTCOME |  | RESIDUAL RISK RATING AFTER EXISTING CONTROLS | ADDITIONAL CONTROLS REQUIRED? | RESIDUAL RISK RATING AFTER ADDITIONAL CONTROLS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Potential Harm | Likelihood |  |  |  |
| Pool Covers: Injury through incorrect use or contact with the structures | Customers and Staff | - Minimum of 1 person operation - Ensure the pool cover ropes/tapes are tucked away - Pool covers are only used overnight - New pool covers were installed for the 2025 season and staff trained on how to operate them correctly | Minor | Possible | 9 |  | 9 |
| Adverse weather: Being an outdoor pool then we can be subject to adverse weather | Customers and Staff | - In adverse weather conditions (thunderstorm /lightning etc) the Lifeguards can order the pool to be evacuated. - In heavy rain, Lifeguards need to move around the poolside more often to minimise the effect of surface distortion caused by the rain and be able to see the pool bottom. - Dry robes and umbrellas have been provided for use by lifeguards when on poolside duty | Minor | Possible | 9 |  | 9 |

| HAZARD AND RELATED ACTIVITIES | PERSONS AT RISK | EXISTING CONTROLS | POSSIBLE OUTCOME |  | RESIDUAL RISK RATING AFTER EXISTING CONTROLS | ADDITIONAL CONTROLS REQUIRED? |  | RESIDUAL RISK RATING AFTER ADDITIONAL CONTROLS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Potential Harm | Likelihood |  |  |  |  |
| Deep Water / Shallow Water supervision: Customers (swimmers and non-swimmers falling in or poor swimmers getting into difficulty | Customers | - The floor of the swimming pool has a tiled band indicating the end of the shallow water area - Water depth signs are displayed around the swimming pool - Close supervision on poolside by qualified LIfeguards | Minor | Possible | 9 |  | Fit a safety barrier in the narrow section by the Pump Room and the deep water end – to be carried out at the next schedule improvements. | 9 |
| Insufficient lighting: Lights not working in the changing rooms | Customers and Staff | - All changing areas are well lit - All broken lights are replaced immediately as part of the daily pool maintenance - As an outdoor pool which is usually only open during daylight hours there is always natural light - External lighting outside the changing rooms is sufficient to light up the bottom of the pool. - During evening/night events, additional lights can be lowered into the pool to light up the pool further. | Insignificant | Possible | 4 |  |  | 1 |

| HAZARD AND RELATED ACTIVITIES | PERSONS AT RISK | EXISTING CONTROLS | POSSIBLE OUTCOME |  | RESIDUAL RISK RATING AFTER EXISTING CONTROLS | ADDITIONAL CONTROLS REQUIRED? | RESIDUAL RISK RATING AFTER ADDITIONAL CONTROLS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Potential Harm | Likelihood |  |  |  |
| Poolside equipment: Pool housekeeping, means that staff do not clear away equipment that is not in use | Customers and Staff | - The only items permitted on poolside whilst not in use is the Lifeguards safety aids - The pool side is cleaned daily to prevent any build-up of debris/litter | Insignificant | Possible | 4 |  | 1 |
| Waterfall feature installed near the kiosk. There is a risk of customers sustaining minor injury from trips and slips around these feature. | Customers | - The waterfall feature is adjacent to the kiosk, was disconnected before the 2025 season so not risk to customers from slips around the feature. | Minor | Possible | 9 |  | 9 |
| Solar panels have been installed at the start of the 2025 season on the boiler house roof (replacing the splash play feature) and along the decking (replacing the seating area there). There is a risk people may try and access those areas and damage the solar panels. There is a risk of injury to customers from unauthorised access to these areas. There is a risk to staff accessing these areas for cleaning and maintenance. | Customers and staff | - The solar panels are fenced off to prevent unauthorised access. - The pool is surrounded by a high fence which is regularly checked and maintained. The fence is fit for purpose. - CCTV cameras are in operation around the site to discourage access. - Cleaning and maintenance around the solar panels only done at supervised times. | Minor | Possible | 9 |  | 9 |
| Cold water swimming sessions were introduced in September 2022. There is a risk of hypothermia and ‘after-drop’. This risk is heightened for those customers new to cold water swimming activity. | Customers and staff | - A specific separate Risk Assessment document for cold water sessions has been produced and is available for each customer when booking a session and placed in a prominent position at the kiosk entrance. - Only lifeguards willing to acclimatise to cold water temperature will work these sessions. - Air and water temperature are displayed clearly. - Lifeguards monitor swimmers closely and are instructed to ask swimmers to exit the pool if they have any concerns about a swimmer’s condition. - It is a condition of entry that swimmers will exit the water on instruction/advice of the lifeguard even if they consider themselves to be fine. | Major | Probable | 20 |  | 20 |

|  | ACTION PLAN (insert additional rows if required) | TO BE ACTIONED B |  | Y | ACTION COMPLETE |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Further control measures identified to reduce risks so far as is reasonably practicable | POSITION | NAME | DATE | SIGNATURE | DATE |
| 1 | NOP and EAP to be reviewed and updated. | Committee Members | Lyn Hill/ML Anderson | Ongoing |  |  |
| 2 | Fit a safety barrier in the narrow section by the Pump Room and the deep water end – to be carried out at the next schedule improvements. | Committee Members | TBC | Ongoing |  |  |
| 3 | Purchase of a portable hoist | Committee member | TBC | Ongoing |  |  |

|  | COMMENTS AND INFORMATION |
| --- | --- |
|  | Risk Assessment is published on the Ingleton Swimming Pool website and is available on request. |

| SCHEDULED DATE OF NEXT REVIEW (MINIMUM ANNUALLY) | ARE THERE ANY CHANGES TO THE ACTIVITY SINCE THE LAST ASSESSMENT? CLARIFY THAT ALL CONTROLS ARE STILL IN PLACE & MONITORED ON A REGULAR BASIS. | SIGNATURE OF MANAGER | DATE OF REVIEW |
| --- | --- | --- | --- |
| April 2020 – by Management Committee before start of the next season | Due to the Coronavirus pandemic in 2020 the pool did not open for the 2020 season |  |  |
| May 2021 – by Management Committee before start of 2021 season | Review of the Accident Book for the last operating season (2019) does not indicate any additional risks that need to be included within this Risk Assessment. The Risk Assessment set out above still applies for the 2021 season however a separate Covid-19 Risk Assessment has been prepared for the start of the 2021 season due to Coronavirus restrictions still being in place. Also new for the 2021 season is the introduction of 2 new water features on the premises- consideration of the impact of these is included in the revised Risk Assessment for 2021. It has also been decided wherever possible for the 2021 season to run Member sessions with a lifeguard present. | ENDORSED BY INGLETON POOL MANAGEMENT COMMITTEE 12TH MAY 2021 |  |
| May 2022 – by Management Committee before start of next season | Review of the Accident Book for the last operating season (2021) does not indicate any additional risks that need to be included within this Risk Assessment. The Risk Assessment set out above still applies for the 2022 season. Reference to Covid-19 has been retained as coronavirus is still prevalent in the country, however, all legal responsibilities have been disbanded.It has been agreed that for the 2022 season, only the waterfall feature will be operational. | ENDORSED BY INGLETON POOL MANAGEMENT COMMITTEE 18TH MAY 2022 |  |
| May 2023 – by Management Committee before start of next season | Review of the Accident Book for the last operating season (2022) does not indicate any additional risks that need to be included within this Risk Assessment, but addendum to Risk Assessment put in place for the trialling of cold water swimming from September 2022. The Risk Assessment set out above still applies for the 2023 season. Reference to Covid-19 can be removed. It has been agreed that for the 2023 season, that both water features will be operational, with the splashzone feature sessions being limited with an additional lifeguard being present during these sessions. Usage to be monitored and reports to be sent to the committee. Cold water swimming trial from September 2022 was deemed to be successful and will continue outside the main summer (heated) season to extend the use of the pool. | ENDORSED BY INGLETON POOL MANAGEMENT COMMITTEE 19TH APRIL 2023 | 01.04.2023 |
| May 2024 – by Management Committee before start of next season | Review of the Accident Book for the last operating season (2023) does not indicate and additional risks that need to be included within this risk assessment, but addendum to Risk Assessment still in place for cold water swimming (April – May, September – January) to include risks specific to cold water sessions. The risk assessment set out above still applies for the 2023 season. Reference to COVID-19 remains whilst a vaccination programme still in place for vulnerable persons. Splashzone features in operation last year and safety procedures to continue this season – set opening times and an additional Lifeguard to supervise. Need for a portable hoist has been identified and will need to be risk assessed once purchased and training taken place. | ENSORSED BY INGLETON POOL MANAGEMENT COMMITTEE 9th May 2024 | 01.05.24 |
| July 2024 | Amendment for 2024 season endorsed by Ingleton Pool Management Committee – 2 members of staff required for poolside monitoring during Splashzone sessions (one to be a Lifeguard, the second member of staff doesn’t need to be a lifeguard, but must stand by entrance to splashzone whilst splashzone is on) |  |  |
| May 2025- prior to start of the season | The risk assessment set out above applies to the 2025 season. Additional comments added due to installation of solar panels on the premises at the start of the 2025 season. Risks associated with Splash Zone equipment now mitigated as equipment has been removed. Separate Risk Assessment for cold water sessions is noted but now cross-referenced in this main Risk Assessment for completeness. | Reviewed by Rachel Whaley in consultation with members of Ingleton Pool Management Committee | 15.05.2025 |
| May 2026 – in advance of next year’s season | The rist assessment set out above applies to the 2026 season. There has been no significant changes to the operation of the pool. | Reviewed by ML Anderson in consultation with members of Ingleton Pool Management Committee. | 20.05.2026 |